I – V characteristics Data Analysis

When investigating the current – voltage relationship it is conventional to plot the current (the dependent variable) on the y-axis.

For a resistor the relationship between current and voltage is given by:

R 	= 	V    /    I

This can be arranged as:

I 	= 	1     x    V

R

and compared with			y	= 	m          x	+	c

In this case the gradient is not R, but 1 / R.

The following data was produced for the resistance coil of a Rheostat. Work out the average current for each voltage and plot a graph with I/A on the y-axis and V/V on the x-axis.

| V/V | I/A |  |  |  |
| --- | --- | --- | --- | --- |
|  |  |  |  | Average |
| 1.0 | 0.032 | 0.033 | 0.033 |  |
| 2.0 | 0.076 | 0.077 | 0.075 |  |
| 3.0 | 0.123 | 0.125 | 0.125 |  |
| 4.0 | 0.168 | 0.170 | 0.168 |  |
| 5.0 | 0.213 | 0.214 | 0.212 |  |
| 6.0 | 0.260 | 0.257 | 0.259 |  |
| 7.0 | 0.306 | 0.307 | 0.305 |  |
| 8.0 | 0.350 | 0.357 | 0.359 |  |
| 9.0 | 0.395 | 0.395 | 0.396 |  |
| 10.0 | 0.440 | 0.440 | 0.441 |  |

Although the graph should in theory pass through the origin it may have a small intercept due to contact resistance. Taking this into account, draw a best fit line through the points.

Measure and record below the gradient of your graph

This is not the resistance of the resistor – in this case:

Gradient = 1 / R		and so		R = 1 / Gradient

Work out the resistance of the resistor and give a suitable unit.