v

Force

e

Field

1.	(a)	Force = Bev   Diagram 
		Use Fleming's Right Hand Rule to work out the direction of the force,

- but remember conventional current is opposite to electron flow.

[2]

- (b)	(i)	Electrons in wire are being moved at right angles to the field 

- [1]

- (ii)	There is a force on these electrons pushing them towards one end of the wire 

- Electrons will move along the wire until electrostatic repulsion balances the magnetic force

- This displacement of electrons produces a pd across the wire 

- [3]

- (iii)	Length of wire (l) 

- [1]

- (iv)	EMF = Blv 

- [1]

- (c)	EMF = Blv 	= 5.2 10-5  20  300 

- =  0.31V       Remember to give the unit in the final answer

- [2]

- Total 10 marks

- 2.	(a)	Galvanometer pointer deflects as magnet moves in 

- Returns to zero as magnet stops 

- Deflects in opposite direction   same magnitude as magnet removed 

- [4]

- (b)	Solenoid cuts the flux of the magnet 

- Induces emf across solenoid, producing current in galvanometer 

- When magnet stops, rate of cutting flux is zero, emf falls to zero 

- When magnet removed, emf is reversed (Lenz's Law) 

- Same rate of cutting flux so magnitude of emf is the same 

- [5]

- (c)	The deflection of the galvanometer will be greater in each direction 

- Flux is being cut at a faster rate 

- [2]

- Total 11 marks

- 3. 	(a)	B = o n I    where n = number of turns per metre

- B	= 410 -7150/0.6 1.2 

- = 3.810 -4 T = 0.38 mT 

- [3]

- (b)	(i)	Flux linkage through second coil is  = B  area  number of turns

- =  0.38 10-3    (2.510-2) 2  50 

- =  3.7  10-5 Wb 

- Induced emf 	= d/dt = 3.710-5 / 0.1

- = 3.710-4 V 

- [4]

- (ii)	Assumption is that all the flux produced by the solenoid links with the second coil 

- [1]

- (c)	By reversing the current, the change in flux linkage is doubled 

- (It changes from 3.710-5 Wb to -3.710-5 Wb, i.e. 7.410-5 Wb)

- Since the emf is the same, d/dt must be the same 

- So the time taken must also have doubled, i.e. 0.2 seconds 

- [3]

- Total 11 marks

- 4.	(a)	v = 2gs =   (29.810.4)  = 2.8ms-1 

- This question is mainly about induction, but be prepared to use other

- parts of physics when needed.

- [2]

- (b)	EMF across the side moving into the field is Blv	= 0.250.52.8 
 				= 0.35V 

- Total resistance of frame is 2 

- I = V / R = 3.5 / 2 = 0.175A 

- [4]

- (c)	Force on wire due to field  = BIl  = 0.250.1750.5 = 0.022N upwards

- Total force acting downward on frame 	= mg - 0.022  

- = 0.019.81 - 0.022

- = 0.0761N 

- Acceleration = F/m = 0.0761 / 0.01 = 7.61ms-2    downwards 

- [5]

- Total 11 marks

- 5.	(a)	The direction of an induced emf  is such as to oppose the change producing it 

- [2]

- (b)	(i)	The positive peak shows the emf induced as the magnet enters the coil 

- The rate of cutting flux increases as more of the field enters the coil so the emf increases 
By Lenz's Law, the current in the coil produces a magnetic field to try and repel the magnet 

- [3]

- (ii)	As the magnet begins to drop out of the coil the emf becomes negative 
Again by Lenz's Law, the current in the coil must reverse to try and stop the magnet leaving the coil 

- [2]

- (iii)	Since the magnet is accelerating under gravity, it is moving faster as it leaves the coil 
The rate at which flux is cut is increased, so the emf, shown by the size of the negative peak, is greater 

- [2]

- (iv)	The magnet is moving faster as it falls out of the coil, the duration of the negative peak is shorter than that of the positive peak 

- [1]

- Total 10 marks

- 6.	(a)	The self inductance of a coil is the ratio of the induced emf  across it to the rate of change

- of current through it 

- Alternatively in symbols:  L = V / (dI/dt)

- [2]

- (b)	(i)	Current = 3.0A so the pd across the resistance is IR = 3.00.5 = 1.5V 

- Voltage across L is 12 - 1.5 = 10.5V 

- L = V/ (dI/dt) = 10.5 / 60 = 0.175H or 175mH 

- [3]

- (ii)	At switch on there is no current immediately, so the whole battery voltage (12V)

is across the inductor 
dI/dt = V / L = 12 / 0.175 = 69As-1 



- [3]

- (iii)	After several seconds the current will have risen to a steady value 

- It is no longer affected by L, so it can be found by I = V / R
I = 12 / 0.5 = 24A 

- [2]

(c)

I (A)

t (s)

- correct axes     exponential shape of curve 

- [2]

- Total 12 marks

- 7.	(a)	(i)	Hold search coil so that it is at right angles to field 

- Twist coil sharply through 180o to reverse the field through it 
EMF (or current) induced is proportional to field strength 

- [3]

- (ii)	Apparatus: to measure induced current use a sensitive galvanometer or ballistic galvanometer 

- [1]

- (iii)	Measure: deflection of galvanometer  at different distances from wire
Repeat measurements and average 

- [3]

- (iv)	Errors: reading galvanometer deflection (moving) 
position of coil (held by hand) 
difficulty of twisting coil at same rate each time 

- [3]

(b)

B

(galvanometer readings proportional to B)

B  1/r

distance r (m)

- graph   relationship 

- [2]

- (c)	B = oI / 2r = 410-72 / (20.1)  = 410-6 T or 4T

- [2]

- Total 14 marks

- 8.	(a)	(i)	B is flux density, i.e. flux per unit area  so:  = Barea 

- [2]

- (ii)	Units: B tesla (T)   weber (Wb)  which is Tm2

- [2]

- (b) 	(i)	Area swept out by radial line in one revolution is area of disc 	= r2 

- = (0.2510-2)2  
flux cut = Barea = 0.1(0.2510-2)2   = 2.010-6Wb 

- [4]

- (ii)	This flux is cut 25 times per second
emf = d/dt = 2.010-6 / (1/25)  = 510-5V  

- [2]

- (iii)	EMF across diameter is zero 
Opposite emfs across two radial lines cancel out

- [1]

- Total 11 marks

- 9.	(a)

Input voltage Vp

Primary coil 
(Np turns) 

Output voltage Vs 

Secondary coil 
(Ns turns) 

Laminated soft iron core 

- Step up transformer, so Vs > Vp  Diagram 

- [8]

- (b)	Faraday's Law states: induced emf is proportional to rate of change of flux linkage 

- AC in the primary coil produces a varying magnetic flux in this coil 

- This flux is linked to the secondary coil by the iron core 

- Changing flux through secondary coil induces voltage across it 

- [4]

- Total 12 marks

- 10.(a)	(i)	Flux  = Barea  = 0.12(110-2)2 = 1.210-5Wb 

- emf = rate of change of flux = 1.210-5 / 0.60 = 2.010-5V 
current = V/R = 2.010-5 / 2.310-3 =  8.710-3A = 8.7mA 

- [4]

- (ii)	Energy = powertime = IVt   	= 8.710-32.010-50.60  
 	= 1.0410-7J 

- [3]

- (b)	(i)	Loop should have a larger area (e.g. 30cm30cm) 
 	and should be wound with many turns (e.g. 1000) 

- [2]

- (ii)	Earth's field is small, greater area and more turns give flux linkage that will produce a measurable current 

- [1]

- (iii)	Loop is mounted so it can be rotated  about horizontal and vertical axes.

- This allows horizontal and vertical components  of field to be measured

- [2]

- Total 12 marks

- 11.	(a)	Currents induced  in a piece of metal due to changing flux or movement through

- a magnetic field  Eddy currents circulate around paths of least resistance in metal 

- [3]

- (b)		Eddy currents can be large and cause the metal to become hot.

- Application: inductive heating e.g. for melting metals 

- Disadvantage: energy loss in core of wound components e.g. transformers 

- [2]

- (c)	Kinetic energy of pendulum is converted into heat energy 

- by eddy currents induced in the vane. Energy loss causes pendulum to slow down 

- [2]

- (d)		Oscillations last for longer time 

- Eddy currents are smaller due to slots increasing resistance, so there is less energy loss

- [2]

- Total 9 marks

- 12.	(a)	Call the voltages and the number of turns on the primary and secondary coils: 	Vp , Vs, Np, Ns .

Faraday's Law for the secondary coil:  Vs = - Ns (d/dt) 

In order to maintain current in the primary, the input voltage Vp must equal the back

emf due to the changing flux 
So Faraday's Law also gives: Vp = - Np (d/dt) 

Assume all the flux through the primary passes through the secondary,

so that  is the same in each equation, then: - (d/dt) = Vs / Ns = Vp / Np  

Thus:  Vs / Vp = Ns / Np 

[5]

- (b)		(i)	Turns ratio Ns / Np = Vs / Vp  = 6 / 240 = 1 / 40 

[2]

- (ii)	Power supplied = 24W at 240V
Current = P / V = 24 / 240  = 0.1A 

[2]

- (iii)	This assumes that no power is lost in the transformer, i.e. the transformer is perfect 

[1]

Total 10 marks

- 13.(a)	Connect search coil to a CRO 

- Place coil between turns of slinky so its plane is at right angles to the field 

- Measure amplitude of induced emf displayed on CRO 

- Repeat measurement at different points along the slinky 

[4]

- (b)	(i)	B  I   so twice the current gives twice the field strength 

- This would double the amplitude of the CRO trace.

- [2]

- (ii)	B  n, where n is the number of turns per metre. Stretching the slinky to twice its length halves the turns per metre  (since total number of turns remains the same).

- Thus B is halved 

- [2]

- (iii)	Since one change doubles B and the other halves it, the two together would leave B unchanged 

[1]

- (c)	Either: search coil and galvanometer, twist coil to induce current 

- or: Hall probe with voltmeter to measure Hall voltage  field 

[1]

Total 10 marks

- 14.(a)	(i)	The spokes of the wheel cut magnetic flux 

- This induces an emf across each spoke, i.e. between the hub and the rim 

- [2]

- (ii)	Change: the speed of rotation, the strength of the field, the orientation of the wheel in the field (any two) 

[2]

- (b)	(i)	The bulb would light  and then go out as the current decreased 

- [2]

- (ii)	The wheel would slow down  as energy is transferred to heat and light in the bulb 

[2]

- (c)	Since emf is 1/4, the rate of cutting flux must be 1/4 that of the first wheel 

- This is Barea swept out per second 

- B and rate of rotation are the same, so area of wheel must be 1/4 i.e. radius is halved 

[3]

Total 11 marks

15.(a)

Rotating  coil 

N

S

Magnetic field 

Current taken via slip rings  

Diagram 

[5]

- (b)	Faraday's Law: emf is proportional to rate of change of flux linking the coil 

- Coil is rotated at a steady rate, flux through coil changes as angle of coil to field changes 
On each half turn the polarity of the emf changes (side of coil that was rising is now falling so direction of current reversed) 
Output is a continuous alternating voltage 
Contact via slip rings allows current to be collected while maintaining coil rotation 

[5]

- (c)	Replace slip rings with a commutator  to reverse connections on each half turn

- so that polarity of output remains constant 

[2]

Total 12 marks

- 16.(a)	(i)	Flux linkage through coil,  	= Bareanumber of turns  

- = 5.210-53.410-41000 = 1.76810-5Wb 
Flux changes to zero in 0.3s
emf = rate of change = 1.76810-5 / 0.3 = 5.910-5V 

[3]

- (ii)	Current = V/R = 5.910-5 / 45 = 1.310-6A 

[1]

- (b)	The change in flux linkage is doubled 

- The induced emf is multiplied by 4, i.e. rate of change of flux linkage is  4 

- The time taken must be halved, i.e. time = 0.15s 

[3]

I

t (s)

1

2

(c)	(i)

Correct axes   two cycles shown 

[2]

- (ii)	Maximum current 	= BAN2f / R 
		= 5.210-53.410-4100021 / 45 
		= 2.510-6A = 2.5 A 

- [3]

- Total 12 marks

- 17.	(a)	Connect coil to a galvanometer 

- push magnet into coil, observe deflection 

- remove magnet from coil, observe deflection in opposite direction 

- Induced current produces a field that opposes the motion of the magnet 

- [4]

- (b)	As coil is pushed into magnet, work is done against repulsion due to field of coil 

- Kinetic energy of the magnet is converted into electrical energy in the coil and then to KE

- in the meter 

- As coil is removed, work is done against attraction of field of coil 

- The same energy conversion takes place 

- In each case Lenz's Law predicts the direction of induced current consistent with the energy

- transfer  The opposite direction would allow energy to be created from nothing 

- [6]

- Total 10 marks

- 18.(a)	(i)	dI/dt  = V / L 

- Initially, all supply voltage is across inductor, so dI/dt  = 5 / 5  = 1As-1 

- [3]

- (ii)	When I = 0.3A the pd across R is IR = 0.310 = 3V
So the voltage across L is 5 - 3 = 2V 
 dI/dt  = V / L = 2 / 5 = 0.4As-1 

- [2]

- (iii)	After 5s the current will have reached a steady value  limited only by the resistance,

- I = V / R = 5 / 10 = 0.5A 

- [2]

- (b)	(i)	Changing the value of R will not affect the initial rate of change of current

- (supply voltage / L) 

- [1]

- (ii)	I = 0.3A , IR = 0.31 = 0.3V

- V across L = 5 - 0.3 = 4.7V 
dI/dt  = V / L = 4.7 / 5 = 0.94As-1

- Steady current after 5s, I = V / R = 5 / 1 = 5A 

- [2]

- Total 10 marks

- 19.	(a)	(i)	Ring is repelled by the coil and floats above it on the iron rod 

- Ring becomes hot due to current circulating in it 

- [2]

- (ii)	Current in coil is changing (AC)

- This induces a current in the ring 
This current opposes the change producing it, as its field repels the field of the coil 

- [2]

- (b)	(i)	Hold magnet with one pole near rim of disc 
 	Move magnet round the disc, just above it. The disc follows the magnet 

[2]

- (ii)	Movement of magnet induces a current in the disc 
The field due to this current opposes the change producing it, i.e. the magnet moving away 
Thus the field of the disc is attracted to the magnet and the disc follows it 

[3]

Total 9 marks