# 3.4 Environmental Pollution

# Atmospheric Pollution

Syllabus content:

Atmospheric Pollution

Atmospheric pollution behaves very differently to water or ground pollution as it moves very rapidly by the wind and so can be carried over large areas; can interact with electromagnetic radiation from the sun; can interact with Infrared radiation from the earth.

Acid Rain

Natural acidity of rain

Normal unpolluted rainfall is naturally acidic (pH 5.6) as carbon dioxide will dissolve to produce a dilute solution of carbonic acid.

Acid rain includes snow, sleet, fog, cloud water and dew.

Enhanced Acidity

Acid rain usually refers to enhanced acidity produced by human activities. The table below summarise the main causes of acid rain.

| GAS | ACTION | MAJOR SOURCES |
| --- | --- | --- |
| Sulfur dioxide (gas) | Dissolves to form sulfurous acid (weak) | Combustion of coal & oil |
| Sulfur trioxide (gas) | Dissolves to form sulfuric acid (strong) | Oxidation of sulfur dioxide by ozone |
| Oxides of nitrogen (gas) | Dissolves to form nitric acid (strong) | High temperature combustion in power stations & petrol and diesel combustion engines. |
| Ozone (tropospheric) | Secondary pollutant that oxidises sulfur dioxide to sulfur trioxide | Photochemical reactions involving oxides of nitrogen & oxygen |
| Hydrogen chloride (gas) | Dissolves to form hydrochloric acid | Combustion of coal and incineration of wastes containing chlorine (e.g. PVC plastic waste) |

Task:

Research the effects on acid rain and summarise your findings on a poster.  Ensure that you indicate whether you effects are direct or indirect.  Attach the poster to this booklet.

General strategies to control Pollution

Critical pathway analysis

Used to predict the movement of pollutants and to plan monitoring programmes.  It can’t be assumed that any discharge will be diluted and dispersed.  In the atmosphere and water it can become re-concentrated.  (Think & link – water cycle)

Mainly used for movement of radioactive materials released from a nuclear power station or waste processing site.  Ensures that sites at most risk are sampled

Factors to take into account:

Properties of the pollutant

State of pollutant (s,l,g)

Density

Solubility in water/lipids

Chemical stability

Wind speed and direction

Features of the environment

Geology – porosity /permeability

Effect of pH and O2 on solubility

Temperature

Ctitical group monitoring

____________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Emission control strategies

Control of emission location

  - Some choices may affect the severity of the pollution caused:

    - Marine discharges where water currents will dilute and disperse emissions.

    - Emissions downwind of urban areas

    - Not discharging waste onto permeable rock above an aquifer

Control of emission timing

  - Tidal cycle

    - emissions into incoming tide will carry upstream.  Emission into outgoing tide will disperse them

  - Temperature inversions

    - Atmospheric emissions in a temperature inversion will not disperse.  Restrict emissions in cities where temperature inversions are common

Principles of pollution control

The polluter pays principle:

____________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

The precautionary principle:

____________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Selection of control

| Control method | Example |
| --- | --- |
| Production prevention | Desulfuristion of fossil fuels |
| Prevention of release | Electrostatic precipitators for smoke control Catalytic converters for NOx, CO and hydrocarbons |
| Post-release prevention | Oil spill clean-up methods Phytoremediation of land contaminated with heavy metals |
| Alternative processes | Use of electric vehicles instead of diesel or petrol Using pyrethroid pesticides instead of more polluting organochlorides (e.g. DDT) Using renewable energy sources |

Efficiency of pollution control

Explain the economic principle behind the cost of removing pollutants using the graph.

______________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Control of Acid Rain

Control of oxides of sulfur

Ways to reduce sulfur dioxide in waste gases.  For each method explain how it works and include any relevant equations

- Natural gas desulfurisation:

- Crude oil desulfurisation:

- Coal desulfurisation:

- Dry flue-gas deslfurisation (dry FGD)

Label the diagram below.

What waste product is produced by dry flue gas desulphurisation and can it be used for?

- Wet flue-gas desulfurisation (wet FGD)

Explain the basic details of wet flue gas desulphurisation and label the diagram below.

What waste product is produced by dry flue gas desulphurisation and can it be used for?

Control of oxides of nitrogen

Ways to reduce nitrogen oxides (NOx) in waste gases.  For each method explain how it works and include any relevant equations

- Low temperature combustion

- Catalytic converters

- Urea sprays

Smoke Pollution and Smogs

Smoke Smogs

Smog is an interaction between smoke and fog.

Fog is formed when moist air is cooled so water vapour condenses forming a layer of airbourne droplets of water.

Fog + smoke = Smog

The great London smog of 1052 – Case Study

- During the smog in London in 1952 over 4000 people died of respiratory problems in a single week.  This led to the Clean Air Act (1956) being introduced.

- This allowed local authorities to ban the release of excessive smoke and the burning of smokey fuel in urban smokeless zones where smoke control orders had been made.

Watch the video about the great London smog and in the space below make notes on the causes and effects during this time.

Control of Smoke Pollution

Ensure you have explained how each of the following control measures reduce smoke pollution:

Legislation:

____________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Domestic sources:

____________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Transport sources:

____________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Industrial sources:

Electrostatic precipitator:

____________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

__________________________________________________________________________________________________________________________________________________________________

Cyclone separator:

________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Scrubber:

________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Coal treatment:

____________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Bag filters:

____________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Tropospheric ozone and Photochemical Smogs

Photochemical smogs involve a wide variety of reactions between primary and secondary pollutants in the presence of sunlight including UV light

Tropospheric ozone

Tropospheric ozone is a secondary pollutant produced by chemical reactions with primary pollutants released due to human activities

Effects of ozone:____________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Photochemical smog

Like smoke smogs, these are caused by a build up of atmospheric pollutants, often when there is a temperature inversion.  They involve the chemical reactions of vehicle exhaust emissions during sunny conditions, usually with a temperature inversion that causes the pollutants to become more concentrated.  The word ‘smog’ is misused here are photochemical smogs do not involve smoke or fog.

Formation of photochemical smogs

Equations

Write the equation below that are involved in producing PANs and explain how they occur:

Explanation of how PANs are formed:

________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Effects of PANs

________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Control of photochemical smogs

Summarise below the main ways in which photochemical smogs can he reduced.

________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Research

  - Which cities tend to be affected? What do they have in common?

  - What are the effects on people and living things?

  - Can you find a news story to illustrate these ideas?

Lichens as Biotic Indicators of Pollution

Research:

- What are lichens?

- What basic types of lichen are there?

- What is a biotic index?

- What features of a species are needed to be a good biotic indicator?

- How can lichens be used to measure pollution?