- 1.	(a)	An electron of charge e moves with velocity v at right angles to a 	magnetic field

- of strength B.

Field

e

v

- Write down a formula for the force on the electron and add an arrow to the above

- diagram to indicate the direction of this force.

- [2]

- (b)	A straight wire is now moved with a velocity v, at right angles to the magnetic field, of

- strength B, as shown:

B

v

wire

- (i)	How is this situation similar to that in part a)?

[1]

- (ii)	Explain how this leads to an induced emf across the wire.

- [3]

- (iii)	What additional information do you need in order to calculate the induced emf?

- [1]

1.	(b)	(iv)	Write down the formula for the induced emf.

[1]

- (c)	An aeroplane with a wingspan of 20m flies horizontally at a speed of 300ms-1.

- If the Earth’s vertical component of magnetic flux density is 5.2  10-5 T, calculate the

- emf induced between its wingtips.

[2]

- Total: 10 marks

- 2.	A solenoid is connected to a sensitive galvanometer. A bar magnet is pushed quickly into the

- solenoid and then removed at the same speed.

- (a) 	State the effect you would expect this action to have on the galvanometer.

- [4]

- (b)	Explain these observations.

[5]

- (c) 	The action is now repeated at a higher speed. State and explain what difference you would

- expect to see in the response of the galvanometer.

[2]

- Total 11 marks

- 3.	(a)	Calculate the intensity of the magnetic field inside a 150 turn air cored solenoid of length

- 60 cm, carrying a current of 1.2 A.

- Take the permeability of free space o as 4 x 10 -7 Hm -1.

[3]

- (b)		A second coil of 50 turns and diameter 5 cm is wound tightly on the solenoid.

- The current in the solenoid is switched off and falls to zero in 0.1s.

- (i)	Calculate the emf induced across the second coil.

- [4]

- (ii)	What assumption did you make in this calculation?

- [1]

- (c)	In another experiment the current in the solenoid is reversed rapidly. The emf across the

- second coil is found to be the same as in b). How long did it take to reverse the current?

- [3]

- Total 11 marks

- 4.	A closed loop of wire has a total length of 2m, a mass per unit length of 5g m-1 and a resistance of 1 m-1. It is formed into a square frame and held with its plane vertical, above the poles of a large magnet, which provides a horizontal field of 0.25T.

- The frame is released and falls between the poles, travelling 40 cm before the lower edge enters the field. Take g as 9.81 ms-2.

N

S

- Calculate the following quantities at the moment the lower edge enters the field:

- (a)	The speed of the frame

- [2]

- (b)	The current in the frame

- [4]

- 4.	(c)	The magnitude and direction of the acceleration of the frame

[5]

Total 11 marks

- 5.	(a)	State Lenz's Law of electromagnetic induction.

- [2]

- (b)	A magnet is dropped vertically through a coil of wire. A CRO connected to the coil displays

- a trace of the emf induced in the coil.

N

S

To CRO

emf

mV

time ms

Explain the following features of the trace:

- (i)	The positive peak

- [3]

- (ii)	The negative peak

[2]

- (iii)	The relative magnitudes of the two peaks

- [2]

- 5.	(b)	(iv)	The duration in time of the two peaks

- [1]

- Total 10 marks

- 6.	(a)	Define self inductance.

- [2]

- (b)	A 12 volt battery of negligible internal resistance is connected in series with a resistance of 0.5 and a coil of self inductance L. When the circuit is switched on the current increases

- from zero. At a certain time the current is 3.0A and is increasing at 60A s-1.

0.5 

- Calculate:

- (i)	The value of L

- [3]

- (ii)	The rate of increase of the current when the circuit is first switched on

[3]

- (iii)	The value of the current after several seconds

[2]

- 6.	(c)	Sketch a graph showing how the current varies with time during the few seconds after

- the circuit is switched on.

- [2]

- Total 12 marks

- 7.	A long straight vertical wire has a direct current of 2A flowing in it.

- (a)		Describe how a search coil could be used to investigate the magnetic field at different distances from the wire. Include in your description:

- (i)	The principal of operation of the search coil when used with a d.c. field

- (ii)	Other apparatus you would require

- (iii)	The measurements you would take

- (iv)	Three sources of error in the measurements

- [10]

- (b)	When the measurements are completed, they are plotted on a graph of field strength

- against distance from the wire. Make a sketch of the shape of this and state the

- relationship between field and distance that it indicates.

- [2]

- 7.	(c)	Taking o as 4 x 10-7 Hm-1 ,calculate the value of the field you would expect to find at

- a distance of 10cm

- [2]

- Total 14 marks

- 8.	(a)	(i)	State the relationship between magnetic field strength, B, and  flux, 

- [2]

- (ii)	State the SI units of each quantity.

- [2]

- (b)	A conducting disc of diameter 5cm is mounted with its plane perpendicular to a uniform

- magnetic field of 0.1T. It rotates at 25 revolutions per second.

B

A

O

- (i)	Calculate the flux cut by a radial line AO as the disc makes one revolution.

- [4]

- (ii)	Hence calculate the emf induced between the centre of the disc and a point on its rim.

- [2]

- 8.	(b)	(iii)	What is the emf induced between the end points of a diameter of the disc?

[1]

- Total 11 marks

- 9.	(a)	Draw a fully labelled schematic diagram of a step up transformer.

- Label the input and output voltages and indicate their relative size.

- [8]

- (b)	Use Faraday's Law to explain why a voltage is produced across the output of a

- transformer when AC is applied to its input.

- [4]

- Total 12 marks

- 10.(a)	A closed wire loop is made into a square of side 1.0cm and mounted vertically with its plane

- perpendicular to a magnetic field of 0.12T. The resistance of the loop is 	2.3  10-3.

- The field is reduced to zero at a uniform rate in 0.60s.

- Calculate the:

- (i)	current induced in the loop as the field collapses.

- [4]

- (ii)	energy dissipated in the loop as the field changes.

[3]

- (b)	A square wire loop can be used to measure the magnetic field of the Earth.

- (i)	State two ways this loop would differ from the one described above.

- [2]

- (ii)	Explain why this difference is necessary.

- [1]

- 10.(b)	(iii)	Describe how the loop should be mounted in order to make the measurements.

- [2]

- Total 12 marks

- 11.(a)	What is meant by the term 'eddy currents'?

- [3]

- (b)	State one application and one disadvantage of eddy currents.

- [2]

- (c)	An aluminium vane as shown below is allowed to oscillate like a pendulum between

- the poles of a magnet.

- Explain why the vane slows down and stops after a very few swings.

- [2]

(d)	The vane is replaced by one with slots in it as shown.

- State and explain what difference this would make to the oscillations.

- [2]

- Total 9 marks

- 12.(a)	Use Faraday's Law of electromagnetic induction to derive the relationship between the turns ratio of a transformer and the input and output voltages.

- [5]

- (b)	A step down transformer reduces a 240V AC supply to 6V in order to power a toy train

- with a motor rated at 24W.

- (i)	What is the turns ratio of this transformer?

- [2]

- (ii)	What current flows in the primary coil?

- [2]

- (iii)	What assumption did you make in answering (ii) ?

- [1]

- Total 10 marks

- 13. A long slinky spring is connected to an AC supply of constant amplitude and frequency.

- (a)	Explain how a search coil may be used to measure the magnetic field inside the slinky.

- [4]

- (b)	State and explain how the measurement would be altered if the following changes were 	made:

- (i)	Double the current in the slinky

- [2]

- (ii)	With the original current, stretch the slinky to twice its length

- [2]

- (iii)	Both changes i) and ii) together

[1]

- 13.(c)	If the AC supply were replaced by a DC one, what method could be used to measure

- the field?

[1]

- Total 10 marks

- 14. A bicycle wheel is mounted vertically on a metal axle in a horizontal magnetic field.

- Sliding contacts are made to the metal rim of the wheel and the axle.

- The wheel is set to rotate freely.

Magnetic field

Contacts

- (a)	(i)	Explain why an emf is generated between the contacts.

- [2]

- (ii)	State two ways in which this emf could be varied.

- [2]

- (b)	A small light bulb is connected between the contacts. State and explain what you would

- observe about:

- (i)	The light bulb

- [2]

- (ii)	The rotation of the wheel

- [2]

14. (c)	A second wheel is mounted in the same way at the same orientation to the field and

rotated at same angular speed. The emf measured is ¼ that of the first wheel.

What do you conclude about this wheel?

[3]

- Total 11 marks

- 15.(a)	Draw a fully labelled diagram of a simple AC generator.

- [5]

- (b)	Use Faraday's Law of electromagnetic induction to explain the operation of the generator.

- [5]

- (c)	State a modification that would have to be made if the generator is to produce a

- unidirectional output voltage.

[2]

- Total 12 marks

- 16. A coil of 1000 turns and resistance 45 has a cross sectional area of 3.4cm2.

- It is connected in series with an ammeter of negligible resistance.

- (a)	The coil is mounted with its plane perpendicular to the Earth's magnetic field and is

- rotated through 90o, so that it is parallel to the field, in a time of 0.3s.

- The magnetic flux density of the Earth's field at this location is 5.2  10-5T.

Calculate:

- (i)	the average emf induced in the coil

- [3]

- (ii)	the average current recorded on the ammeter

[1]

- (b)	In a second experiment the coil is rotated through 180o. The average current recorded was

- 5.2 10-6A. Calculate the time taken to rotate the coil.

- [3]

- (c)	It is now arranged to rotate the coil continuously at 1 revolution per second.

- (i)	Sketch a graph showing how the current varies over 2 seconds.

- [2]

- 16.(c)	(ii)	Calculate the maximum value of the current.

[3]

- Total 12 marks

- 17.(a)	Describe an experiment to demonstrate Lenz's Law using a bar magnet and a solenoid.

- What other equipment would you need?

- [4]

- (b)	Describe the energy changes that take place during the experiment and show that 	Lenz's Law

- is consistent with energy conservation.

[6]

- Total 10 marks

- 18. A circuit consists of an inductor L of 5H with negligible resistance in series with a resistor R

- of 10 and a DC supply of 5.0V.

5.0 v

10 

5 H

- (a)	Calculate:

- (i)	the initial rate of change of the current when the circuit is switched on

- [3]

- (ii)	the rate of change of the current when the current is 0.3A

[2]

- (iii)	The current after about 5s

- [2]

- (b)	The resistance is now replaced by 1.

- (i)	Which of the three quantities calculated above will be unaffected?

- [1]

- 18.(b)	(ii)	Calculate new values for the other two quantities.

- [2]

- Total 10 marks

- 19. A coil of 2000 turns is placed over an iron retort stand rod as shown.

- A 12V AC supply is connected to the coil.

Iron rod

2000 turn coil

12V AC

Aluminium ring

- (a)	The aluminium ring is dropped onto the rod.

- (i)	Describe what happens to the ring.

- [2]

- (ii)	Explain how your observations demonstrate Lenz's Law.

[2]

- (b)	An aluminium disc is mounted horizontally and is free to rotate.

Disc

Magnet

19.(b)	(i)	State how you would cause the disc to rotate using a bar magnet without coming into

contact with the disc.

- [2]

- (ii)	Use Lenz's Law to explain your method.

- [3]

- Total 9 marks