Consider the circuit loop consisting of the cell and the 3.0  resistor:

1.6 V = 3 I1 + 2  (I1 + I2).

Thus: 1.6 V = 5  I1 + 2  I2.

Consider the circuit loop consisting of the cell and the 6.0 resistor:

1.6 V = 6  I2 + 2  (I1 + I2).

Thus 1.6 V = 2  I1 + 8  I2.

Subtracting the second equation from the first gives:

0 V = 3  I1 + 6  I2

hence I1 = 2 I2.

Substituting I1 = 2 I2 into the second equation gives:

1.6 V = 12  I2.

Thus I2 = 0.13 A and I1 = 0.27 A.

Current through cell = I1 + I2 = 0.40 A.

pd across 3.0 resistor= I1 × 3.0     (= I2  6.0  ) = 0.8 V.

M1.(a)     I3 = I1 + I2 ✓

1

(b)     10 V ✓

1

(c)     I2 = (12 – 10) / 10 ✓

Allow ce for 10 V

1

= 0.2 A ✓

The first mark is for the pd

The second is for the final answer

1

(d)     pd across R2 increases

As R1 increases, pd across R1 increases as pd = I1 R1 ✓

First mark is for identifying that pd across R1 increases (from zero).

1

pd across R3 = 10 V – pd across R1

Therefore pd across R3 decreases ✓

Second mark is for identifying that pd across R3 must decrease

1

pd across R2 = 12 – pd across R3

Therefore pd across R2 increases ✓

Third mark is for identifying that this means pd across R2 must increase

1

[7]

M2.(a)     (i)      (use of I = V / R)

first mark for adding resistance values 90 k Ω

I = 6.0 / (50 000 + 35 000 + 5000)     = 6.7 × 10−5A

accept 7 × 10−5 or dotted 6 × 10−5
but not 7.0 × 10 −5 and not 6.6 × 10 −5

2

(ii)     V = 6.7 × 10−5 × 5000   = 0.33 (0.33 − 0.35) V  
OR
V = 5 / 90 × 6    = 0.33( V)

CE from (i)
BALD answer full credit
0.3 OK and dotted 0.3

2

(b)     resistance of LDR decreases

need first mark before can qualify for second

reading increase because greater proportion / share of the voltage across R OR higher current

2

(c)     I = 0.75 / 5000 = 1.5 × 10−4 (A)   
(pd across LDR = 0.75 (V))
pd across variable resistor = 6.0 − 0.75 − 0.75 = 4.5 (V)   
R = 4.5 / 1.5 × 10−4 = 30 000 Ω  
or
I = 0.75 / 5000 = 1.5 × 10−4 (A)  
RtotalI = 6.0 / 1.5 × 10−4 = 40 000 Ω  
R = 40 000 − 5000 − 5000 = 30 000 Ω

3

[9]

M3.(a)    potential divider formula used or current found to be 0.25 A

C1

A1

allow 1 s.f.

2.0 V

1.0 V (with working) gains 1 mark

2

(b)     main current =1.2 V / 4 Ω = 0.3 (A)

C1

Rtotal = 1.8 V / 0.3 A = 6 Ω or I8 = 0.225 (A)

C1

RV = 24 Ω

A1

3

[5]

M4.          (a)     (i)      6.0 (Ω) (1)

1

(ii)     4.5 (V) (1)

1

(iii)     (use of I = V/R)

I = 4.5/6.0 = 0.75 (A) (1)

current through cell A = 0.75/2 = 0.375 (A) (1)

2

(iv)    charge = 0.375 × 300 = 112 (1) C (1)

2

(b)     cells C and D will go flat first or A and B last longer (1)

current/charge passing through cells C and D (per second) is
double/more than that passing through A or B (1)

energy given to charge passing through cells per second is double
or more than in cells C and D (1) or in terms of power

3

[9]

M5.(a)     (i)      I =  = 0.80 A (1)

(ii)     P = (0.80)2 × 5 = 3.2 W (1) (allow e.c.f. from (a)(i))

(2)

(b)     Itot =  (1) = 1.60 (A) (1)

I =  = 0.80 (A) (1) (allow e.c.f. from Itot)

(3)

(c)     same brightness (1) 
because same current (1) 
[or an answer consistent with their current values]

(2)

[7]