| Unit 1 | EMF and Internal Resistance |  |
| --- | --- | --- |
| Lesson 18 |  |  |
| Learning Outcomes | To know what emf and internal resistance are |  |
|  | To know how to measure internal resistance |  |
|  | To be able sketch and interpret a V-I graph, labelling the gradient and y-intercept | N. DWYER |

# Energy in Circuits

In circuits there are two fundamental types of component: energy givers and energy takers.

# Electromotive Force (emf), ε

Energy givers provide an electromotive force, they force electrons around the circuit which transfer energy.

The size of the emf can be calculate using:                  ε=EQ

This is similar to the equation we use to find voltage/potential difference and means the energy given to each unit of charge. We can think of this as the energy given to each electron.

The emf of a supply is the p.d. across its terminals when no current flows

EMF is measured in Joules per Coulomb, JC-1 or Volts, V

Energy takers have a potential difference across them, transferring energy from the circuit to the component.

emf = energy giver                               p.d. = energy taker

Energy is conserved in a circuit so energy in = energy out, or:

The total of the emfs = The total of the potential differences around the whole circuit

# Internal Resistance, r

The chemicals inside a cell offer a resistance to the flow of current, this is the internal resistance on the cell.

Internal Resistance is measured in Ohms, Ω

## Linking emf and r

If we look at the statement in the box above and apply it to the circuit below, we can reach an equation that links emf and r.

Total emfs = 	total potential differences

ε	= 	(p.d. across r) 	+  (p.d. across R)              {Remember that V=IR}

ε	=	  (I x r)	      +	       (I x R)

ε	=	Ir	      +	      IR

ε = I(r+R)

The terminal p.d. is the p.d. across the terminals of the cell when a current is flowing

ε       =   internal p.d   +  terminal p.d.

So the above equation can be written as ε = Ir + V  where V is the terminal p.d.

# Measuring emf and r

We can measure the emf and internal resistance of a cell by measuring the current and voltage as shown on the right, the variable resistor allows us to get a range of values. If we plot the results onto a graph of voltmeter reading against ammeter reading we get a graph that looks like the one below.

Graphs have the general equation of y = mx+c, where y is the vertical (upwards) axis, x is the horizontal (across) axis, m is the gradient of the line and c is where the line intercepts (cuts) the y axis.

If we take ε = Ir + V and arrange it into y= mx + c

y axis = V   and   x axis = I

ε = Ir + V       V = -Ir + ε             V = -r I + ε

y  =m x +c

So we can see that the:

y-intercept represents the emf

and

gradient represents (–)internal resistance