TAP 118- 2: Tapping off a potential difference

A series circuit is connected as shown in the diagram.

1.	What is the potential difference between A and B?

2.	An additional resistor of 100  is connected between the 50 resistor and the cells. What is the potential difference between A and B now?

3.	The additional 100  resistor is now connected in parallel with the first 100  resistor. What is the potential difference between A and B now?

4.	A potential divider is made from a 4 k and a 6 k resistor connected in series with

a 20 V supply. Draw a diagram of the arrangement. What four values of potential difference can be tapped off?

5.	A student puts a 12  variable resistor in series with a 6 V battery, expecting to get

a variable potential difference.

The voltmeter is a high resistance digital multimeter. Explain why the circuit won't work. Draw a circuit which would work.

6.	B is the wiper of a 100  rotary potentiometer.

What is the full range of the potential difference that can be tapped off between A and B?

Hints

1.	Resistors are in the ratio 50 :100 so the potential difference splits up 1 : 2.

3.	Work out the equivalent resistance of the two 100  resistors in parallel first.

6.	Find the pd set up across the potentiometer first by looking at the ratio of the resistors.

Answers and worked solutions

1.	100  / (50  + 100 ) x 6 V = 4 V

2.	100  / (50  + 100  + 100 ) x 6 V = 2.4 V

3.	Resistance of parallel combination = 50 . So the 6 V supply potential difference splits equally between the two 50  and pd across AB is 3 V.

4.	Resistance of whole potential divider is 10 kW

p.d. across 4 kΩ = 4 kΩ10 kΩ×20 V=8 V

p.d. across 6 kΩ = 6 kΩ10 kΩ×20 V=12 V

p.d. across 10 kΩ = 10 kΩ10 kΩ×20 V=20 V

5.	The resistance has been connected as a variable resistor. The multimeter has an extremely high resistance so that wherever one moves the sliding contact the pd is set up across the voltmeter which always reads 6 V! The redrawn diagram should show the ends of the resistance connected across the battery

6.	The resistance of whole potential divider is 400 . The supply pd 12 V splits between the fixed 300  resistor and the 100  potentiometer in the ratio of their resistances. So there will be 9 V across 300  and 3 V across 100 .

When the slide contact from B is next to A the pd tapped between A and B is 0 V; when the slide is at the other end of the potentiometer the full pd of  3 V is across AB.

External references

This activity is taken from Advancing Physics Chapter 2, 170S