4.12.4 Components and their Characteristics 1

Lamp and resistor in series

Study the characteristics of the two electrical components A and B shown.

1.	What is the resistance of each component with a potential difference of 3 V across it?

2.	Suggest what each component could be.

The two components are connected in series across a variable d.c. supply. A high resistance digital voltmeter measures the pd across A.

3.	What is the current through A when the voltmeter reads 3 V?

4.	What is the potential difference being provided by the supply?

5.	If the supply potential difference is increased so that the voltmeter reads 6 V, what is the power being dissipated in each component?

Hints

1.	Read the currents flowing with a potential difference of 3 V

3.	Read the current from the graph.

4.	Read the pd across B at a current equal to that through A; add the pd to that across A.

5.	Find the current in A from the graph; find the pd across B for this current from the graph; use P = I V

Answers and worked solutions

1.	A = 12 ; B = 7.5 

2.	A is an ohmic resistor, perhaps a metal wire. The resistance of B increases with current. It could be a filament lamp.

3.	0.25 A

4.	4.5 V

5.	3 W

External references

This activity is taken from Advancing Physics, Chapter 2, 190D