- 1.	Assume the cell in the circuit has negligible internal resistance.

V2

R1=3

R2=2

R3=6

V1

I1

I2

I3

V3

9V

Calculate:

  - the resistance of the parallel combination (R2 and R3)

[2]

  - the total resistance of the circuit

[1]

  - the potential difference across each resistor

    - V1 =

[1]

    - V2 =

[1]

    - V3 =

[1]

- 1.	(d)	the current flowing through each resistor

- (i)	I1 =

[1]

- (ii)	I2 =

[1]

- (iii)	I3 =

[1]

Total 9 marks

- 2.	(a)	Define resistivity.

[2]

- (b)	Calculate the resistance of a 20cm length of copper wire which has a diameter of 1mm.

- The resistivity of copper is 1.610-8

[3]

- (c)	The copper wire is now stretched to three times its original length. Its volume is unchanged. Calculate its new resistance.

[2]

  - Explain why copper wire is rarely used to make high resistance resistors.

[2]

Total 9 marks

- 3.	(a)	You wish to measure the internal resistance of a dry cell.

- (i)	Draw the circuit diagram and label the equipment you would use.

- [3]

- (ii)	Describe the measurements you would make and explain how these would be used to calculate the internal resistance.

[3]

- (b)	A 10 and a 12 resistor are connected in series with a battery of emf 6V and internal resistance 2. Calculate:

    - the total current in the circuit

[1]

    - the pd. across the 10 resistor

[1]

    - the pd across the battery terminals

[2]

- 3.	(b)	(iv)	the power developed in the 12 resistor

[2]

Total 12 marks

- 4.	(a)	(i)	Explain why there is often a difference between the emf of a cell and the 	voltage measured

- across its terminals.

[2]

- (ii) 	Under what circumstances are these values the same?

[1]

- (b)	A battery of emf 9.0V is connected across a 32 resistor.

- If the pd across the resistor is 8.0V, calculate:

- (i)	the current in the circuit.

[1]

- (ii)	the internal resistance of the battery.

[2]

- (iii)	the current flowing in the circuit if a second, identical, battery were added in series in

- the circuit reinforcing the emf of the first.

[3]

Total 9 marks

- 5.	The diagram represents a typical potential divider circuit. Assume the internal resistance of the

- supply is negligible and the resistance of the voltmeter is infinite.

V

10

20

12V

- (a)	Calculate the voltmeter reading.

[2]

- (b)	The 10 resistor is removed and replaced by a 40 resistor. Calculate the potential

- difference across the 40 resistor.

[2]

- (c)	Explain why the voltmeter reading has changed.

[2]

- (d)	The voltmeter is now replaced with one of 100 resistance. State and explain, without

- further calculation, what will happen to the voltmeter reading.

[2]

Total 8 marks

- 6.	(a)	A light bulb of resistance 600 is powered by a 240V supply. Calculate:

- (i)	the current flowing through the filament of the bulb

[1]

- (ii)	the quantity of charge which passes through the bulb in 10 minutes

[1]

- (iii)	the energy dissipated by the bulb in 10 minutes.

[2]

- (b)	(i)	Sketch the arrangement of three identical bulbs that will produce the maximum power

- output.

- [1]

- (ii)	If they are connected to the supply in (a), what is the power output for each bulb?

[3]

- Total 8 marks

- 7.	(a) A pd of 6.0V is applied across a 2m length of copper wire of diameter 0.1mm. Calculate:

6V

2m

Copper wire

- (i)   the resistance of the wire

[3]

- (ii)  the current flowing through the wire

[1]

- (iii)  the drift velocity of the electrons in the wire

[3]

- (b) The diagram shows the lattice of copper irons in a wire. Illustrate on the diagram a typical

- path taken by an electron moving through the lattice.

+

-

[2]

Total 9 marks

Data: Density of free electrons in copper = 1.010 29m-3

Resistivity of copper = 1.710 -8 m

Electron charge = 1.610 -19C

- 8.	An electric current, I, passes through a conductor of cross sectional area A.

- (a)	Show, with the aid of a diagram, that:

I = nAve

- where e is the charge on one electron, n is the number of free electrons per cubic metre

- and v is the drift velocity

[6]

- (b)	Three different wires of the same length and cross-sectional area are connected in series as shown. Charge flows through the wire.

A

B

C

- The densities of free electrons in the materials are:

| nA | nB | nC |
| --- | --- | --- |
| 1.0  10 29 m-3 | 5.0  10 28 m-3 | 1.0  10 28 m-3 |

- (i)	In which wire will the electrons be moving fastest? Explain your answer.

[2]

- (ii)	Which wire would you expect to have the least resistance? Explain your answer.

[2]

Total 10 marks

- 9.	(a)	Define the following electrical terms and state the SI units in which they are measured:

- (i)	potential difference

[2]

- (ii)	electromotive force

[2]

- (iii)	current

[2]

    - resistance

[2]

    - power

[2]

- (b) Using the definitions you have stated above, show that each of the following combinations

- of quantities are equivalent to power:

- (i) potential difference  current

- [1]

- 9.	(b)	(ii)	(potential difference)2 / resistance

[1]

- (iii) (current)2  resistance

[1]

Total 13 marks

- 10.	The light dependent resistor in the circuit below has a resistance of 600 in the dark and

- 120 in sunlight.

150

10V

V

- (a) Calculate the voltmeter reading

- (i) in the dark

[2]

- (ii) in sunlight

[1]

- (b)	(i) What assumption did you have to make about the voltmeter in answering  part (a)?

[1]

- (ii) Explain what difference it would make to the results if this assumption were not true.

[2]

- 10.(c)	Describe one practical use for this circuit.

[2]

Total 8 marks

- 11.The circuit shows a voltmeter with a resistance of 5K connected across a 20k resistor.

V

20k

20k

12V

- (a) 	Calculate:

- (i)	the voltmeter reading

[3]

- (ii)	the current through the voltmeter

[2]

- (b) 	Repeat the calculations in (a) taking the resistance of the voltmeter as 50k

- (i)

[3]

- (ii)

[2]

- 11.(c)	What does this calculation demonstrate about the resistance required for a voltmeter?

[2]

Total 12 marks

- 12. The diagram / circuit shows a network of four resistors.

X

Y

Z

W

6

3

6



- (a) 	For the network shown above, calculate the resistance between:

- (i)	X and Y

[3]

- (ii) 	X and Z

[3]

- (b) 	Determine the current in the 3 resistor if a 12V supply were connected between X and Z

[2]

Total 8 marks

- 13.(a) 	A 90cm length of wire with a cross sectional area of 310-7cm2 is connected to a 6.0V

- battery. When the circuit is switched on, a current of 4A is measured in the wire.

900 cm

wire

6V

- Calculate

- (i) the resistivity of the wire

[3]

- (ii)	the conductivity of the wire

[2]

- (b)	Another wire of the same metal has twice the diameter and half the length of the first one.

- Calculate:

- (i) 	the resistance of this wire

- [2]

- (ii) 	the current when it is connected to a 6.0V battery

[2]

- 13.(c)	State which of the two wires, (a) or (b) dissipates the more power.

- Explain your answer without calculation.

- [2]

- Total 11 marks

- 14.(a) 	The voltmeter shown in the circuit below reads 3V when connected, as shown, to an

- 8V supply with negligible internal resistance.

V

800

1200

8V

Calculate the resistance of the voltmeter

[5]

- (b) The voltmeter is now removed and one of half the resistance is placed across the 1200

- resistor. Calculate its reading.

[4]

Total 9 marks

- 15.(a) 	Explain what is meant by the following terms in describing an electric cell:

- (i) Internal resistance

[2]

- (ii) Terminal voltage

- [2]

- b)	In the circuit below, the ammeter has a resistance of 0.5 and the voltmeter, which can be considered to have an infinite resistance, reads 1.0V.

V

A

5

emf = 1.5V

- Calculate the internal resistance of the cell shown in the above circuit.

[3]

Total 7 marks

- 16.The circuit below can be used as a thermometer.

RTH

V

R

EMF = 9.0V

| Temp/ oC | RTH  | V / V |
| --- | --- | --- |
| 0 | 800 |  |
| 20 | 400 | 1.8 |
| 60 | 100 | 4.5 |
| 100 | 20 |  |

R is a fixed resistor of 100. The 9V cell has negligible internal resistance and the voltmeter has very high resistance.

The table shows the value of resistance, RTH at certain temperatures.

- (a) 	The third column shows the voltmeter reading at different temperatures.

- Calculate the missing values and add these to the table.

[2]

- (b) 	Plot a graph of voltmeter reading against temperature.

- [3]

- 16.(c)	At a certain temperature the voltmeter reads 3.0 V.

- Use your graph to obtain an approximate value of:

- (i) 	the temperature.

[1]

- (ii) 	the resistance of RTH at this temperature

[2]

Total 8 marks

- 17. A student sets up this circuit in order to demonstrate Ohm's Law using a 5 resistor.

V

5

A

Variable Resistor RV = 0 - 10

Using different values of the variable resistor, he obtains the following results:

| - RV ( | - I (A) | - V (V) |
| --- | --- | --- |
| - 2 | - 2.3 | - 7.5 |
| - 4 | - 1.6 | - 5.5 |
| - 6 | - 1.3 | - 4.3 |
| - 8 | - 1.1 | - 3.5 |
| - 10 | - 0.9 | - 3.0 |

- (a)	Plot the graph the student would use to demonstrate Ohm's Law.

- [3]

- (b) Is the graph what you would expect if the resistor obeyed Ohm's Law?

- Explain  your answer.

[2]

- 17.(c)	Is the graph what you would expect for a 5 resistor? Explain your answer.

[2]

- (d) 	What does this result tell you about the equipment used? Explain your answer with a

- calculation.

[3]

- (e)	Both the battery and the ammeter have negligible resistance.

- Calculate the emf of the battery.

[2]

Total 12 marks

- 18.(a) 	The tungsten filament in a light bulb changes its resistance as its temperature rises.

- (i)	Sketch a graph of I against V to illustrate this variation.

- [3]

- (ii)	Explain this behaviour in microscopic terms.

[3]

- (b)  A negative temperature coefficient thermistor also shows a variation of resistance with

- temperature.

- (i) How does this variation differ from that of the filament?

[1]

- (ii) 	Explain this difference in terms of the structure of the material from which

- the thermistor is made.

[3]

Total 10 marks

- 19.An electric motor is used to raise a load.

A

When the switch is closed, the ammeter reads 30mA and the motor raises a 500g load through a height of 75cm in 12 seconds. Take g = 9.8 ms –2.

- (a) Assuming the motor to be 100% efficient and the internal resistance of the battery to be

- negligible, calculate the EMF of the battery.

[3]

- (b) In fact the EMF is 12V and the motor is 88% efficient.

- Calculate the internal resistance of the battery.

[5]

- Total 8 marks