4.12.4 Components and their Characteristics - Semiconductor devices 1

Apparatus:

- LDR, e.g. NORP-12

- Multi-meter (to use as an ohm- meter)

- Lux meter

- 12 V filament lamp in holder (e.g. SBC batten holder)

- Power supply.

- Black card

- Scissors

- Sellotape

Procedure:

- Set up a 12 V lamp on the bench to act as a light source.

- Connect the multimeter to the LDR and adjust it to a suitable resistance range.

- Now place the LDR at different distances from the lamp and measure its resistance.

- At each distance replace the LDR by the lux meter and record the light intensity.

- You should end up with a table showing distance, light intensity and resistance.

Plot a graph to show how the resistance (y-axis) varies with light intensity (x-axis).