4.13.2.1 More about Resistance - Resistor Combination HW

Simplifying circuits

These are questions about replacing many resistors with one resistor which draws the same current from the cell. Study the circuit diagrams and try to simplify sections of the circuit by putting in an equivalent value resistor. Redraw the diagram for each step until you are reduced to one equivalent resistor before calculating the current. Many of these problems are easier if you think about conductance rather than resistance.

For each circuit find the current drawn from the power source.

1.

2.

3.

In this circuit calculate:

4.	The current through the 6  resistor.

5.	The potential difference across the 12  resistor.

Hints

1.	Start with the parallel combination.

2.	Start with the series combination.

3.	Tackle each parallel combination first.

4.	Current through resistor = branch current = pd / total branch resistance.

5.	Calculate the branch current and then pd = resistor value x current through it.

Answers and worked solutions

1.	0.8 A

2.	2.5 A

3.	2 A

4.	0.44 A

5.	3.2 V

1.	Circuit current = 0.8 A   (Equivalent resistor for parallel arrangement = 2.4 W; total circuit resistance = 7.4 W)

2.	Circuit current = 2.5 A (Equivalent resistor for series arrangement = 8 W; Total circuit resistance = 4.8 W) or current = 12 V / 12 W + 12 V / 8 W = 2.5 A

3.	Circuit current 2 A = (Equivalent resistor for the parallel arrangements: 2.35 W and

3.53 W; Total circuit resistance = 5.88 W)

4.	I = 4 V / (3 W + 6 W) = 0.44 A

5.	pd = 3.2 V (I 12 W = 4 V / (12 W + 3 W) = 0.27 A; V12 W = 0.27 A x 12 W = 3.2 V)

External references

This activity is taken from Advancing Physics Chapter 2, 130S

Resistor series

Resistors are manufactured in limited values. You will find that the numbers range from 1.0 to 10 in 24 steps, each differing from the next by about 10%. i.e.:

| 1.0 | 1.5 | 2.2 | 3.3 | 4.7 | 6.8 |
| --- | --- | --- | --- | --- | --- |
| 1.1 | 1.6 | 2.4 | 3.6 | 5.1 | 7.5 |
| 1.2 | 1.8 | 2.7 | 3.9 | 5.6 | 8.2 |
| 1.3 | 2.0 | 3.0 | 4.3 | 6.2 | 9.1 |

Two or more of these resistors can be combined to give other values of resistance.

Finding useful combinations.

In the laboratory there are resistors with the values 1 k, 2.2 k, 3.3 k, 4.7 k, 5.6 k and

6.8 k

How can you combine two or more of these resistors when you need a resistance of:

1.	3 k

2.	9 k

3.	500 

4.	5 k

5.	4 k

Practical advice

Questions for practice and gaining confidence. Questions 4 and 5 may require trial and error. Encourage students to estimate the effect of placing resistors in parallel

An alternative strategy is to consider the conductance required.

Answers and worked solutions

Possible combinations

1.	Three 1.0 k in series.

2.	6.8 k and 2.2 k in series.

3.	1 k and 1 k in parallel.

4.	Two 5.6 k in parallel then connected in series with a 2.2 k.

5.	Parallel combination of two sets of 3.3 k in series with 4.7 k

External references

This activity is taken from Advancing Physics Chapter 2, 140S