- 1.	(a)	1 / Rparalllel = 1 / R2 + 1 / R3 =  4 / 6  

- Rparallel  = 6/4 = 1.5

[2]

- (b)	Rtotal = 1.5 + 3 = 4.5

[1]

- (c)	The supply voltage is divided in proportion to resistance.
9V is divided 2:1 between series resistor (3and parallel combination (1.5)

    - V1 = 6V  

- [1]

    - V2 = 3V  

- [1]

    - V3 = 3V  

[1]

  - The current in each resistor is given by Ohm’s Law, I = V/R

- (i)	I1 = V1 / R1 = 6 / 3 = 2A  

[1]

- (ii)	I2  = V2 / R2 = 3 / 2 = 1.5A  

[1]

- (iii)	I3 = V3 / R3 = 3 / 6 = 0.5A  

[1]

Total 9 marks

- 2.	(a)	ρ= RAL

- where R = resistance (); A = cross section  of area (m2); L = length of conductor (m) 

- [2]

- (b)	Cross sectional area A = (d/2)2 = (1  10 –3 /2)2 = 7.86  10 –7m2  
R = L / = 1.6  10 –8  0.2 / 7.8610 –7  = 4.07  10 –3 

[3]

- (c)	Volume is fixed so increasing length by a factor of 3 will decrease cross sectional area by a

- factor of 3 
R increases by a factor of 9  
New R = 9  4.0710 –3 = 3.6610-2

[2]

  - Resistivity is very low  
So wire would need to be very long and/or very thin  

[2]

- Total 9 marks

V

R

E

r

Cell

Variable

resistor 

3.	(a)	(i)

- Diagram

Voltmeter

- [3]

(ii)	Measure terminal voltage with no current  being drawn (switch open) 	to give emf, E

i.e. v = - Ir + E

Close switch and measure terminal voltage, V, with current being drawn

Calculate internal resistance r = R(E – V)/V 

[3]

(an alternative, and generally better, method is to measure current  and terminal pd

for a range of variable resistor settings.  A graph of is plotted as shown below

V

I

gradient = -r)

gradient = -r

- (b)	(i)	I = V/R = 6/24 = 0.25A 

[1]

- (ii)	V = IR = 0.25  10 = 2.5V 

[1]

- (iii)	V = emf – pd across internal resistance = E – Ir  = 6 – 0.25  2 = 5.5V

[2]

- (iv)	P = I2R = 0.252  12   = 0.75W 

[2]

Total 12 marks

- 4.	(a)	(i)	The terminal voltage is generally less than the emf 

because voltage is lost as current flows through the internal resistance

[2]

(ii)	terminal voltage = emf if no current is drawn from the cell 

- [1]

(b)	(i)	I = V / R = 8.0 / 32 = 0.25A 

[1]

- (ii)	r = (E – V) / I = (9-8) / 0.25  = 4 

[2]

(iii)	Total resistance is now: 32 + 4 + 4 = 40 
Total emf is now:  9 + 9 = 18V 
I = V / R = 18 / 40 = 0.45A 

[3]

Total 9 marks



5.	(a)	R20 ΩRTotal I × 12 = 2030 × 12 = 8v



- [2]

- (b)	40resistor now has two thirds  of supply voltage i.e. 8V 
or: V = 12  40 / 60  = 8V 

- [2]

- (c)	Potential divider equation: V1 = Vsupply  R1 / (R1 + R2) 
A change in value of either of the resistors will affect the pd across the other

- [2]

- (d)	Voltmeter reading will decrease  Voltmeter in parallel with 20 reduces the resistance in

- this arm of the potential divider 

- [2]

- Total 8 marks

6.	(a)	(i)	I = V / R = 240 / 600 = 0.4A 

[1]

(ii)	Q = It = 0.4  600 = 240C 

[1]

(iii)	P = IV = 0.4  240 = 96W 
Energy = P  time = 96  600 = 57600J 

- [2]

- (b)	(i)

- 

- [1]

(ii)	Rtotal = 600 / 3 = 200
P = 2402 / 200 = 288W 

Pone bulb =  288/3 = 96 W 

[3]

- Total 8 marks

7.	(a)	(i)	RL / A
 			0.05  10 -37.85 10-9m2 
 			R =  1.710 -8  2 /   = 4.3 

[3]

- (ii)	I = V / R = 6 / 4.3  = 1.4A 

[1]

- (iii)	v = I / nAe  = 1.4 / (1  1029  7.86  10-91.6  10-19)  = 1.1  10-2ms-1

[3]

- (b)	Random path through the lattice
General drift from negative to positive
e.g.:

+

-

[2]

Total 9 marks

Length L

- 8.	(a)

Area
A

n electrons per m3

Velocity v

- Diagram 

Volume of conductor = AL  

Number of free electrons in conductor = n  volume = nAL  

Quantity of charge free to move = number of electrons  charge on each = nALe 

Current, I = charge / time = nALe 

but  L /t = v I = nAve

[6]

(b)	(i)	The electrons will have greatest velocity in C 

It has the smallest n and therefore the largest v, since I, A and e are constant

i.e. . v = I/nAe      I 1/n

- [2]

- (ii)	A will have the least resistance 
It has the most charge carriers per unit volume 

- [2]

- Total 10 marks

- 9.	(a)	(i)	The potential difference between points A and B is the work done to move a unit charge

- from B to A. 

- SI unit: Volt 

- [2]

(ii) 	The emf of an energy source is the work done per unit charge in moving the charge from its low potential terminal to its high potential terminal

- SI unit: Volt 

- [2]

- (iii)	Current is the flow of charge per second.

- SI unit: Amp

- [2]

- (iv)	Resistance is potential difference between two points resulting current between the two points

- SI unit: Ohm 

- [2]

- (v)	Power is rate of electrical energy transfer = charge transferred through a potential difference per unit time. 

- SI unit: Watt

- [2]

- (b)	(i)	I  V = Qt´V= power 

- [1]

- (ii)	V2R=V´VR=V´I= power 

- [1]

- (iii)	I2R = I  IR = I  V = power 

- [1]

- Total 13 marks

- 10.(a)	(i)	Supply pd is divided across the components in proportion to their 	resistance 

- VLDR = (600  10) / 750 = 8V 

[2]

(ii)	LDR now has a pd of (120  10) / 270 = 4.4V 

[1]

(b)	(i)	It is necessary to assume that the voltmeter has infinite resistance 

- [1]

(ii)	If this is not true, the resistance of the voltmeter across the LDR will lower the total parallel resistance  giving a smaller pd 

[2]

(c)	sensible use: e.g. light meter 
basic description, e.g. the voltmeter reading indicates light level 

[2]

Total 8 marks

11.(a)	(i)	Total resistance of parallel combination:
 	1 / Rtotal = 1 / 20 + 1 / 5 = 5 / 20 
 	Rtotal = 20 / 5 = 4k	Notice it is easier to do this calculation in k

 	Supply pd is divided in proportion to resistance:
 	V = 12  4 (20 + 4) = 2V 

- [3]

(ii)	I = V / R = 2 / 5000  =  4  10 –4A 

- [2]

(b) (i)	Parallel combination: 1 / Rtotal = 1 / 20 + 1 / 50  = 7 / 100
Rtotal = 100 / 7 = 14.3k
V = 12  14.3 / (20 + 14.3) = 5.0V 

- [3]

(ii)	I = 5 / 50  = 0.1mA 

- [2]

(c)	The voltmeter with higher resistance has less effect on the voltage being measured / draws less current  
Ideal voltmeter would have infinite resistance / resistance should be as large as possible 

- [2]

- Total 12 marks



12.(a) 	(i)	1 / RXY = 1 / 6 + 1 / (6 + 3 + 12) = (7 + 2) / 42 

RXY = 42/9 = 4.7

[3]



(ii)	1 / RXZ = 1 / (6 + 12) + 1 / (3 + 6) = (1 + 2) / 18 
RXZ = 18 / 3 = 6 

[3]

(b)	The arm of the network containing the  resistor has:
3 + 6 = 9 between X and Z 
 = V / R = 12 / 9 = 1.33A 

[2]

Total 8 marks

13.	(a)	(i)	 The resistance of the wire is R = V / I = 6 / 4 = 1.5

 = RA / L = 1.5  3  10 –11 / 0.9 	Notice cm2 converted to m2
 = 5  10 -11m 

[3]



(ii)	 = 1 /  = 1 / 5  10 -11 = 2  1010m-1 

[2]

(b)	(i)	It has 2 diameter and therefore 4 the cross sectional area 	and it has ½ length 

Since R = L / A, the resistance will be 1/8 of the original.
R = 1.5 / 8 = 0.19

[2]



- (ii)	I = 6 / 0.19 = 32 A 

- [2]

(c)	The wire in (b) will dissipate more power 
P = V2 / R, so power increases as resistance decreases with constant voltage

- [2]

- Total 11 marks

14.	(a)	Parallel combination has 3/8 of supply voltage

Thus it has 3/8 of total circuit resistance 

i.e. 5/8 of total resistance = 1200

Total resistance = 1920

Parallel combination resistance = 1920 – 1200 = 720 

Meter resistance, RM is given by:
1 / RM = 1 / 720 – 1 / 800 = (9 – 8) / 7200 = 1 / 7200 
RM = 7200 

- [5]

- (b)	Parallel combination: 1/Rparallel = 1/1200 + 1/3600

- Rparallel = 900

- So parallel combination has 900/(900 + 800) of supply voltage 

- So reading is [900/(900 + 800)]  8V = 4.24V (3 s.f.) 

- [4]

- Total 9 marks

- 15.(a) (i)	Internal resistance is the resistance of the cell 

due to the material, chemicals etc. of which it is formed 

[2]

(ii)	Terminal voltage is the potential difference measured across the terminals of the cell

when the cell is supplying current it is the emf minus the 'lost volts' 

[2]

(b)	1.0V is dropped across the 5resistor
By proportion: 0.1V dropped across the 0.5of the ammeter 
This leaves 0.4V dropped across the internal resistance (lost volts) 
By proportion, this gives the internal resistance, r, as:
r = 0.4  5 = 2

[3]

Total 7 marks

- 16.(a)	The voltmeter readings are given by the potential divider equation:
V = 9  100 / (100 + RTH)
The values are shown in the table:

| Temperature oC | Voltmeter reading V |
| --- | --- |
| 0 | 1.0  |
| 20 | 1.8 |
| 60 | 4.5 |
| 100 | 7.5  |

[2]

Voltmeter reading V

(b)	Correct axes  correct points  smooth line drawn 

Temperature oC

[3]

(c)	(i)	From the graph: temperature = 40oC 

[1]

(ii)	Rearranging the equation in a) gives:

RTH = (9  100 / 3) – 100  = 200 

[2]

- Total 8 marks

17.	(a)	Correct axes  correct points  best fit line accurately drawn 

Current A

Voltage V

- [3]

(b)	Yes 
The graph is a straight line through the origin indicating I  V 

- [2]

(c)	No 
The gradient is 1.8 / 6.0 = 0.3, giving R = 1 / 0.3 = 3.33 

- [2]

(d)	The resistance of the voltmeter is reducing the parallel combination 
1 / 3.3 = 1 / Rmeter + 1 / 5  
1 / Rmeter = 0.3 – 0.2 = 0.1,   Rmeter = 10 

- [3]

(e)	Choose a value of RV from the table.
Total R = 3.33 + RV  
Then find emf from V = I  R 
e.g. RV = 10, R = 13.33, V = 0.9  13.33 = 12V 
(Error in results gives a range of values 11.7 – 12.4V)

- [2]

- Total 12 marks

18. (a)	(i)

I

Correct graph  axes labelled 

V

- curve showing decreasing gradient (increasing resistance) 

- [3]

(ii)	As temperature increases, the lattice vibrations increase 
Electrons suffer more collisions 
Therefore resistance increases 

[3]

(b)	(i)	As temperature increases, the resistance of an ntc thermistor decreases 

- [1]

(ii)	Thermistors are made of semiconductor material 

Supplying energy by raising the temperature releases carriers, which are

otherwise used for bonding  and so lowers the resistance 

[3]

- Total 10 marks

19. (a)	Power = rate of doing work = (0.5  9.8  0.75) / 12 = 0.306W  mght
EMF: V = P / I = 0.306 / 0.03 
EMF  = 10.2V 

[3]

(b)	Actual power supplied = 0.306  100 / 88 = 0.348W 
Terminal voltage = pd across motor = P / I = 0.348 / 0.03 = 11.6V 
'Lost volts' = 12 – 11.6 = 0.4V 
Internal resistance r = 0.4 / 0.03 
r = 13

[5]

- Total 8 marks