| OS n FE homework. | Name:; ________________________. Class:; ________________________. Date:; ________________________. |
| --- | --- |

| Time: | 45 minutes |
| --- | --- |
| Marks: | 37 marks |
| Comments: |  |

Q1.

(a)     One of the roles of an operating system is Resource Management. Give three resource management tasks for an operating system for a stand-alone computer.

1. _________________________________________________________________

2. _________________________________________________________________

3. _________________________________________________________________

(3)

(b)     A second role of an operating system is the provision of a virtual machine.

What does this mean?

___________________________________________________________________

___________________________________________________________________

(1)

(c)     Give three tasks that an operating system for a computer connected to a computer network will have in addition to the tasks of an operating system for a stand-alone computer.

1. _________________________________________________________________

2. _________________________________________________________________

3. _________________________________________________________________

(3)

(Total 7 marks)

Q2.

Newspapers and magazines are advertising many different specifications of personal computer systems. Such advertisements feature monitor, disk drives, processor, main memory and operating system.

(a)     What is the purpose of an operating system?

___________________________________________________________________

___________________________________________________________________

(1)

(b)     What is the function of:

(i)      the processor;

______________________________________________________________

______________________________________________________________

(1)

(ii)     main memory (Immediate Access Store)?

______________________________________________________________

______________________________________________________________

(1)

(c)     The system bus in a computer system is made up of three buses.  Name each bus and give one example of its use.

1. Name____________________________________________________________

(1)

Example____________________________________________________________

___________________________________________________________________

(1)

2. Name____________________________________________________________

(1)

Example____________________________________________________________

___________________________________________________________________

(1)

3. Name____________________________________________________________

(1)

Example____________________________________________________________

___________________________________________________________________

(1)

(Total 9 marks)

Q3.

The tables below show two versions of the same segment of a program.

| Version A |  | Version B |
| --- | --- | --- |
| if x > 0: y = y + 2 else: y = y - 1 |  | 00011100 00110000 00101010 10010010 11101010 00000010 00101100 10010001 |

(a)     Shade in one lozenge to indicate which version, A or B, in the tables above represents object code.

| Version A | Version B |
| --- | --- |

(1)

(b)     Describe two differences between a compiler and an interpreter.

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

(2)

(c)     Explain what intermediate code is and why some compilers will produce intermediate code as the final output.

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

(2)

(Total 5 marks)

Q4.

(a)     What is meant by the term stored-program concept?

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

(2)

(b)     The following registers, listed in alphabetical order, are used in the Fetch-Execute cycle:

•        current instruction register (CIR)

•        memory address register (MAR)

•        memory buffer register (MBR)

•        program counter (PC)

•        status register (SR).

Describe, using full sentences, the steps involved in the Fetch-Execute cycle, and how the registers listed above are used. Your description should cover the fetch, decode and execute phases of the cycle.

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

(6)

(c)     Explain the term low-level language.

___________________________________________________________________

___________________________________________________________________

(1)

(d)     Using the assembly language instruction CMP R2,R3 explain the term opcode.

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

(1)

Instructions that can be used in question parts (e) and (f)

| LDR Rd, <memory ref> | Load the value stored in the memory location specified by <memory ref> into register d. |
| --- | --- |
| STR Rd, <memory ref> | Store the value that is in register d into the memory location specified by <memory ref>. |
| ADD Rd, Rn, <operand2> | Add the value specified in <operand2> to the value in register n and store the result in register d. |
| SUB Rd, Rn, <operand2> | Subtract the value specified by <operand2> from the value in register n and store the result in register d. |
| MOV Rd, <operand2> | Copy the value specified by <operand2> into register d. |
| CMP Rn, <operand2> | Compare the value stored in register n with the value specified by <operand2>. |
| B <label> | Always branch to the instruction at position <label> in the program. |
| B<condition> <label> | Conditionally branch to the instruction at position <label> in the program if the last comparison met the criteria specified by the <condition>. Possible values for <condition> and their meaning are: • EQ: Equal to. • NE: Not equal to. • GT: Greater than. • LT: Less than. |
| AND Rd, Rn, <operand2> | Perform a bitwise logical AND operation between the value in register n and the value specified by <operand2> and store the result in register d. |
| ORR Rd, Rn, <operand2> | Perform a bitwise logical OR operation between the value in register n and the value specified by <operand2> and store the result in register d. |
| EOR Rd, Rn, <operand2> | Perform a bitwise logical exclusive or (XOR) operation between the value in register n and the value specified by <operand2> and store the result in register d. |
| MVN Rd, <operand2> | Perform a bitwise logical NOT operation on the value specified by <operand2> and store the result in register d. |
| LSL Rd, Rn, <operand2> | Logically shift left the value stored in register n by the number of bits specified by <operand2> and store the result in register d. |
| LSR Rd, Rn, <operand2> | Logically shift right the value stored in register n by the number of bits specified by <operand2> and store the result in register d. |
| HALT | Stops the execution of the program. |

Interpretation of <operand2>

<operand2> can be interpreted in two different ways, depending upon whether the first symbol is a # or an R:

•        # - Use the decimal value specified after the #, eg #25 means use the decimal value 25.

•        Rm - Use the value stored in register m, eg R6 means use the value stored in register 6.

The available general purpose registers that the programmer can use are numbered 0 to 12.

(e)     Explain what immediate addressing is and write an example of the use of the MOV assembly language instruction, from the table above, that uses immediate addressing.

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

(2)

(f)     Below is a block of program code, written in a high-level language.

IF X = 5
             THEN B ← 10
           END IF

Write a sequence of assembly-language instructions that would perform the same operations as the program code above. Assume that register R1 currently stores the value associated with X, register R2 stores the value currently associated with B and that register R3 is available for general use, if necessary.

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

(4)

(Total 16 marks)