| ATTENUATE | to reduce the signal level. |
| --- | --- |
| PEAKING FILTER | a filter which boosts a specific band of frequencies. |
| BAND | a single filter within an equaliser. |
| BANDWIDTH | a measure of the width of the frequency range altered by a peaking filter. |
| BOOST | to increase a signal's level using a filter. |
| CENTRE FREQUENCY | the frequency at which a peaking filter applies maximum gain. |
| CUT | to reduce a signal's level using a filter. |
| CUTOFF FREQUENCY | the frequency at which a high-pass or low-pass filter has attenuated the signal by 3dB. |
| EQUALISER | audio processor which uses a combination of different filters to alter the balance of frequencies in an audio signal. |
| FILTER | a circuit which alters the level of a limited range of frequencies. |
| FILTER SLOPE | the gain change per octave of a high-pass or low-pass filter. |
| GAIN | the amount by which a filter circuit alters a signal. Can be positive ('boost') or negative ('cut' or 'attenuation'). |
| GRAPHIC EQUALISER | an equaliser which uses a large number of regularly spaced, fixed-frequency filters, each with an individual gain fader |
| HEADROOM | a measure of the maximum signal level that a given equaliser's circuitry can handle. |
| HIGH PASS FILTER (HPF): | a filter which progressively attenuates frequencies below a certain frequency |
| LOW PASS FILTER (LPF): | a filter which progressively attenuates frequencies above a certain frequency. |
| OCTAVE: | a doubling of the frequency |
| Q VALUE: | a measure of the width of the frequency range altered by a peaking filter |
| SEMIPARAMETRIC EQUALISER: | an equaliser with at least one swept peaking filt |
| SHELVING FILTER: | a filter designed to alter the level of all signals beyond a certain frequency by a userdefinable amount |
| SWEPT FILTER | any filter which allows the user control over its characteristic frequency |
| THIRD-OCTAVE | a regular spacing of the filters in a graphic equaliser, where one filter occurs every third of an octave. |
| Parametric EQ: | An equaliser with at least one swept peaking filter with bandwidth control |

Equalisers  KEY WORDS

| Equaliser Definition |  |
| --- | --- |
| Cut |  |
| Boost |  |
| Q |  |
| Filter |  |
| High-Pass Filter |  |
| Low-Pass Filter |  |
| Shelving Filter |  |
| Peak Filter |  |
| Graphic EQ |  |
| Parametric EQ |  |